Slide - 2

Computer Network is a set of devices connected via communication links. A device can be a computer, a printer or any other capable device sending and/or receiving data generated by other devices on the network.

Data communications are exchanging of data between two devices.

Slide - 3

End device is a source or destination device in a networked system.

Intermediary devices interconnect end devices. These devices provide connectivity and work behind the scenes to ensure that data flows across the network.

Intermediary devices connect the individual hosts to the network and can connect multiple individual networks to form an internetwork (internet).

Slide - 4

Simplex Mode, the communication is unidirectional, as on a one-way street. Only one of the two devices on a link can transmit, other can only receive. (ex. Keyboard and traditional monitors)

Half-Duplex Mode, each device can both transmit and receive, but not at the same time. When one device is sending, the other can only receive, and vice versa. The entire capacity of the channel can be utilized for each direction. (ex: Walkie-talkies)

Full-Duplex Mode, both devices can transmit and receive simultaneously. Signals going in one direction share the capacity of the link with signals going in the other direction.( Ex: Telephone network).

Slide – 5

Point-to-Point connection provides a dedicated link between two devices. The entire capacity of the link is reserved for transmission between those two devices.

Multipoint connection is one in which more than two specific devices share a single link. The capacity of the channel is shared.

Slide – 6

Mesh Topology: every device has a dedicated point-to-point link to every other device.

Advantages: no traffic problem, robust, privacy.

Disadvantages: More cabling, installation & reconnection is difficult.

Star Topology:  Each device has a dedicated point-to-point link only to a central controller, usually called hub. The devices are not directly linked to one another.

Advantages: Less expensive than mesh, robust.

Disadvantages: If hub goes down, whole system is dead.

Bus Topology: It is a multipoint connection. One long cable acts as a backbone to link all the devices in a network.

Advantages: Easy to installation, less cabling than mesh & star.

Disadvantages: Difficult in reconnection and fault isolation.

Ring Topology: Each device has a dedicated point-to-point connection with only the two devices on either side of it. A signal is passed along the ring in one direction, from device to device, until it reaches its destination.

Advantages: Easy to install & reconfigure.

Disadvantages: Unidirectional traffic, If a device disable in the ring can disable entire ring.

Slide – 7

LAN:  is usually privately owned and links the devices in a single office

the Peer-to-Peer(P2P) network model allows each node to function as both a client and server.

Wireless is a term used to describe telecommunications in which electromagnetic waves (rather than some form of wire) carry the signal over part or the entire communication path.

A network interface card (NIC) is a circuit board or card that is installed in a computer so that it can be connected to a network.

A MAC address is given to a network adapter when it is manufactured. It is hardwired or hard-coded onto your computer's network interface card (NIC) and is unique to it.

A local area network (LAN) is a group of computers and associated devices that share a common communications line or wireless link to a server. Typically, a LAN encompasses computers and peripherals connected to a server within a distinct geographic area such as an office or a commercial establishment. Computers and other mobile devices use a LAN connection to share resources such as a printer or network storage.

An intranet is a private network that is contained within an enterprise. It may consist of many interlinked local area networks and also use leased lines in the wide area network. Typically, an intranet includes connections through one or more gateway computers to the outside Internet. The main purpose of an intranet is to share company information and computing resources among employees. An intranet can also be used to facilitate working in groups and for teleconferences.

An extranet is a private network that uses Internet technology and the public telecommunication system to securely share part of a business's information or operations with suppliers, vendors, partners, customers, or other businesses. An extranet can be viewed as part of a company's intranet that is extended to users outside the company.

Network convergence refers to the provision of telephone, video and data communication services within a single network. In other words, one pipe is used to deliver all forms of communication services.

Fault tolerance is the property that enables a system to continue operating properly in the event of the failure of (or one or more faults within) some of its components.

Scalability is the capability of a system, network, or process to handle a growing amount of work, or its potential to be enlarged in order to accommodate that growth.

Quality of Service (QoS) refers to the capability of a network to provide better service to selected network traffic over various technologies, including Frame Relay, Asynchronous Transfer Mode (ATM), Ethernet and 802.1 networks, SONET, and IP-routed networks that may use any or all of these underlying technologies

Bring your own device (BYOD)—also called bring your own technology (BYOT), bring your own phone (BYOP), and bring your own Personal Computer (BYOPC)—refers to the policy of permitting employees to bring personally owned devices (laptops, tablets, and smart phones) to their workplace, and to use those devices to access privileged company information and applications

Power line networking (aka Home Plug), which uses your home's electrical wiring as a wired data network

Wireless broadband (also known as WiFi) is a broadband connection to the internet that is accessed without cables. This is different to 3G or 4G mobile broadband, which uses mobile phone signals. It is the connection between your computer and the router that is wireless, not the actual broadband connection itself.